Copiar o código abaixo para o novo arquivo vhdl.

Library IEEE;
use IEEE.std_logic_1164.all;
use IEEE.numeric_std.all;

Entity Cont4_completo is
	PORT(RST: in std_logic;
	     CLK: in std_logic;
		  Q: out unsigned(3 downto 0);
		  EN: in std_logic;
		  CLR: in std_logic;
		  LD:  in std_logic;
		  LOAD: in unsigned (3 downto 0));
end entity;  
Architecture X of Cont4_completo is
Signal CONT: unsigned (3 downto 0);
Begin
	Process (CLK, RST)
		Begin
		If RST = '1' then
		   CONT <= "0000";
		Elsif CLK' event and CLK = '1' then
		   If CLR = '1'then
				CONT <= "0000";
			Else
				If EN = '1' then
					If LD = '1' then
						CONT <= LOAD;
					Else
						CONT <= CONT+1;
					End IF;
				End If;
			End If;
		End If;
	End process;
	Q <= CONT;
End architecture;

Faltaram as linhas relativas às bibliotecas

Library IEEE;
use IEEE.std_logic_1164.all;
use IEEE.numeric_std.all;

O problema é que o projeto chama-se “cont4”, mas a entidade e arquitetura chama-se “cont4_completo”. Substituir e recompilar.

CONTROL H (replace)

Construir o testbench

Salvar como cont4_tb.vhd

Cabeçalho do arquivo

Library IEEE;
use IEEE.std_logic_1164.all;
use IEEE.numeric_std.all;

Entity cont4_tb is
end entity;

architecture nome_qualquer of cont4_tb is

component cont4 is
	PORT(RST: in std_logic;
	     CLK: in std_logic;
		  Q: out unsigned(3 downto 0);
		  EN: in std_logic;
		  CLR: in std_logic;
		  LD:  in std_logic;
		  LOAD: in unsigned (3 downto 0));
end component;

begin
     DUT: cont4
	port map
	     (CLK => clk,
                    Q   => Q,
                    EN => EN,
                    CLR=> clr
                    LD  => ld
                    LOAD:load);

process
	begin
		clk <= ‘0’;
		wait for 15 ns;
		clk <= ‘1’;
		wait for 15 ns;
	end process;	
end architecture;